|  | ISOLATION POINTS |  | Process Area: |  |  | Line 3 Tempering |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 |  | Machine Description: |  |  | Triulzi Washer |  |  |  |
| NEXT AUDIT DUE February 2025 NEXT AUDIT DUE February 2027 NEXT AUDIT DUE February 2028 NEXT AUDIT DUE February 2029 NEXT AUDIT DUE February 2026 |  |  |  |  |  |  |  |  |  |
| W-1 E-1 |  |  |  |  |  |  |  |  |  |
| LOTO PROCEDURE |  |  |  |  |  |  |  |  |  |
| 1 - Notify Affected Employees of the Machine LOCKOUT |  |  |  |  |  |  |  |  |  |
| 2 - Properly Shut Down Machine |  |  |  |  |  |  |  |  |  |
| 3 - Isolate All Energy Sources |  |  |  |  |  |  |  |  |  |
| 4 - Apply Lockout devises, Locks, and Tags |  |  |  |  |  |  |  |  |  |
| 5 – Verify Total De-energization of all sources (use appropriate arc flash PPE according to NFPA 70e standards) |  |  |  |  |  |  |  |  |  |
| ID |  | Source |  |  | LOTO Device |  | Location | Isolation Method | Verify |
|  |  | PRIMARY SOURCE Electrical (480 V) |  |  | 1 Lock 1 Tag 1 Isolation Device |  | E-1 located on the operator side of the control panel | Turn disconnect switch E-1 to the “OFF” position and apply lock and tag. | Verify Isolation |
|  |  | Water Hazard (Water Valve) |  |  | 1 Lock 1 Tag 1 Valve Isolation Device |  | W-1 located on the operator side of the control panel near the floor | Close water valve W-1 and apply valve isolation device lock and tag. | Check water line isolation. |
| 6 - Notify affected employees once the equipment is safe to operate. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | WHO IS RESPONSIBLE FOR SAFETY……I AM |  |  |  |  |  |

| LOCKOUT APPLICATION STEPS |  |  |  |
| --- | --- | --- | --- |
| # | Step | Instruction | Additional Information |
| 1 | Notify Employees | Notify all affected employees that the machine or equipment will be shut down and locked out. |  |
| 2 | Review Procedure | Ensure that each authorized employee understands the type and magnitude of the energy present, the associated hazards and the proper methods of control. |  |
| 3 | Shutdown Equipment | If the machine or equipment is operating, shut it down by the normal stopping procedure. |  |
| 4 | Isolate Energy | De-activate the energy isolating devices(s) so that the machine or equipment is isolated from the energy source(s). |  |
| 5 | Lockout Controls | Lockout the energy isolating device(s) as indicated on the front of this procedure. |  |
| 6 | Dissipate Energy | Dissipate any stored or residual energy as indicated on the front of this procedure. |  |
| 7 | Verify Isolation | Verify that the energy source has been isolated as indicated on the front of this procedure. |  |

| LOCKOUT RELEASE STEPS |  |  |  |
| --- | --- | --- | --- |
| # | Step | Instruction | Additional Information |
| 1 | Inspect Equipment | Check the equipment to ensure that the components are operationally intact. |  |
| 2 | Check Area | Check the immediate area to ensure that all employees have been safely positioned, and that any nonessential items have been removed. |  |
| 3 | Check Neutral | Verify that operating controls are in neutral. |  |
| 4 | Re-Energize | Remove the lockout device(s) and activate the energy isolating device(s) to re-energize the equipment. |  |
| 5 | Notify Employees | Notify affected employees that the servicing or maintenance is completed and the equipment is ready for use. |  |
| 6 | Startup Equipment | Startup equipment and monitor for several operating cycles to ensure it is functioning properly. |  |

Contact your supervisor or safety department if you have any questions or concerns about the accuracy or effectiveness of this procedure.